By: Gerritt Hong – CCNP

Connecting EIGRP and OSPF Networks With BGP (IPv4 & IPv6)

Purpose

The purpose of this lab was to connect multiple sub-networks that used different routing protocols using BGP. EIGRP and OSPF were used inside their own domains, and BGP was configured between them to allow complete IPv4 and IPv6 connectivity across the entire network.

Background

In the real-world networking industry, there are usually many different routing protocols within a network. Different parts of the network are usually added at different times, which can result in un-uniform traits among them. This is why network engineers are expected to know how to connect these protocols properly.

EIGRP is a routing protocol that is known for fast convergence and scalable routing in a network, it uses DUAL to find probe for backup paths and avoid routing loops. EIGRP is mostly used on networks used by mid to large size cooperations because how beginner friendly it is. It also supports IPv4 and Ipv6 and can allow flexible metric tuning. Metrics must also be carefully defined when an EIGRP route is shared to another protocol, so it can learn the routes properly.

OSPF is a routing protocol that is common in much bigger and structured networks, due to it’s ability to organize routers into areas, which makes scalability easier. Another benefit of using OSPF is that it can calculate the shortest path to a target using link-state information with an algorithm called Dijkstra’s algorithm. OSPF can also support both IPv4 and IPv6 but has a separate process for each one.

BGP, however, is different from both those protocols. BGP was made specifically for scalability and to help with the expansion of a network. BGP exchanges routing information between groups known as Autonomous Systems(AS) and what is used to connect the internet. BGP uses an attribute called AS-PATH to prevent routing loops and help with path selection as well.

BGP can form neighbor adjacencies using TCP, but they aren’t discovered automatically. Missing small details like a neighbor being in the wrong address family can prevent routes from being exchanged. In networks that use multiple protocols BGP often acts as a translator between them, while inner-protocols like OSPF and EIGRP that act inside of their own parts of the network. This allows each section with their own protocol to operate on their own while still exchanging their routing information. Redistribution must be done between BGP and the inner-routing protocols to prevent loops or an incorrect path selection. Proper planning and structure are the most important part of any network.

This lab is an example of a network who applies these concepts. In the network there are two EIGRP sub-networks that are connected through an OSPF sub-network with BGP running between each of them. The network’s configuration has the capacity to support IPv4 and IPv6 at the same time. This configuration reflects as industry-level network design and shows how multiple protocols can run together as one network.

Lab Summary

In this lab I configured six routers with EIGRP, OSPF, and BGP to build a multi-protocol network. EIGRP was used on the left and right sides of the topology, while OSPF area 0 was used in the middle. BGP was configured in-between autonomous systems to pass information between the different domains. IPv4 and IPv6 addressing were applied to all links. Routes were redistributed between protocols so every loopback and LAN network could be reached, and connectivity was ensured using routing tables and other show commands.

Lab Commands

- router eigrp / ipv6 router eigrp – Enables EIGRP for IPv4/IPv6

- router ospf / ipv6 router ospf – Enables OSPFv2/OSPFv3

- router bgp – Enables BGP routing

- neighbor <ip> remote-as <as> – Creates BGP neighbor relationships

- address-family ipv4 | ipv6 + neighbor <ip> activate – Enables neighbors per address family

- network – Advertises networks into a routing protocol

- redistribute – Shares routes between routing protocols

- show ip route / show ipv6 route – Confirms learned routes and route sources

- show ip bgp summary / show bgp ipv6 unicast summary – Confirms BGP sessions and prefixes

- show ip ospf neighbor / show ipv6 ospf neighbor – Confirms OSPF adjacencies

- show ip eigrp neighbors / show ipv6 eigrp neighbors – Confirms EIGRP neighbors

Network Diagram

The topology is divided into three routing domains. The left and right sides use EIGRP, while the center uses OSPF area 0. BGP is used at the boundaries between autonomous systems so information can be exchanged across the entire network.

Configurations

Router1:
version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname Router1

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

no ip domain lookup

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 192.168.100.1 255.255.255.255

interface GigabitEthernet0/0/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/0/1

ip address 192.168.1.1 255.255.255.252

ipv6 address 2001:DB8:1::1/64

ipv6 eigrp 65001

negotiation auto

interface GigabitEthernet0/1/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/1/1

no ip address

shutdown

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router eigrp 65001

router-id 1.1.1.1

network 192.168.1.0 0.0.0.3

network 192.168.100.1 0.0.0.0

no auto-summary

ipv6 router eigrp 65001

eigrp router-id 1.1.1.1

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

login

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

line vty 5 15

login

Router 2:

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname Router2

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

no ip domain lookup

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 192.168.100.2 255.255.255.255

interface GigabitEthernet0/0/0

ip address 192.168.1.5 255.255.255.252

ipv6 address 2001:DB8:2::1/64

ipv6 eigrp 65001

negotiation auto

interface GigabitEthernet0/0/1

ip address 192.168.1.2 255.255.255.252

ipv6 address 2001:DB8:1::2/64

ipv6 eigrp 65001

negotiation auto

interface GigabitEthernet0/1/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/1/1

no ip address

shutdown

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router eigrp 65001

router-id 2.2.2.2

network 192.168.1.0 0.0.0.3

network 192.168.1.4 0.0.0.3

network 192.168.100.2 0.0.0.0

no auto-summary

redistribute bgp 65001 metric 10000 100 255 1 1500

ipv6 router eigrp 65001

eigrp router-id 2.2.2.2

redistribute bgp 65001 metric 10000 100 255 1 1500

router bgp 65001

bgp router-id 2.2.2.2

bgp log-neighbor-changes

neighbor 192.168.1.6 remote-as 65002

neighbor 192.168.1.6 description Router3-IPv4

neighbor 2001:DB8:2::2 remote-as 65002

neighbor 2001:DB8:2::2 description Router3-IPv6

address-family ipv4

network 192.168.1.0 mask 255.255.255.252

network 192.168.1.4 mask 255.255.255.252

network 192.168.100.2 mask 255.255.255.255

redistribute eigrp 65001

neighbor 192.168.1.6 activate

exit-address-family

address-family ipv6

redistribute eigrp 65001

neighbor 2001:DB8:2::2 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

login

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

line vty 5 15

login

Router 3:

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname Router3

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

no ip domain lookup

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 192.168.100.3 255.255.255.255

interface GigabitEthernet0/0/0

ip address 192.168.1.6 255.255.255.252

ip ospf 1 area 0

ipv6 address 2001:DB8:2::2/64

ipv6 ospf 1 area 0

negotiation auto

interface GigabitEthernet0/0/1

ip address 192.168.1.9 255.255.255.252

ip ospf 1 area 0

ipv6 address 2001:DB8:3::1/64

ipv6 ospf 1 area 0

negotiation auto

interface GigabitEthernet0/1/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/1/1

no ip address

shutdown

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router ospf 1

router-id 3.3.3.3

network 192.168.1.4 0.0.0.3 area 0

network 192.168.1.8 0.0.0.3 area 0

network 192.168.100.3 0.0.0.0 area 0

redistribute bgp 65002 subnets

ipv6 router ospf 1

router-id 3.3.3.3

redistribute bgp 65002

router bgp 65002

bgp router-id 3.3.3.3

bgp log-neighbor-changes

neighbor 192.168.1.5 remote-as 65001

neighbor 192.168.1.5 description Router2-IPv4

neighbor 192.168.1.10 remote-as 65002

neighbor 192.168.1.10 description Router4-IPv4-iBGP

neighbor 2001:DB8:2::1 remote-as 65001

neighbor 2001:DB8:2::1 description Router2-IPv6

neighbor 2001:DB8:3::2 remote-as 65002

neighbor 2001:DB8:3::2 description Router4-IPv6-iBGP

address-family ipv4

network 192.168.1.4 mask 255.255.255.252

network 192.168.1.8 mask 255.255.255.252

network 192.168.100.3 mask 255.255.255.255

redistribute ospf 1

neighbor 192.168.1.5 activate

neighbor 192.168.1.10 activate

exit-address-family

address-family ipv6

redistribute ospf 1

neighbor 2001:DB8:2::1 activate

neighbor 2001:DB8:3::2 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

login

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

line vty 5 15

login

Router 4:

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname Router4

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

no ip domain lookup

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 192.168.100.4 255.255.255.255

interface GigabitEthernet0/0/0

ip address 192.168.1.13 255.255.255.252

ip ospf 1 area 0

ipv6 address 2001:DB8:4::1/64

ipv6 ospf 1 area 0

negotiation auto

interface GigabitEthernet0/0/1

ip address 192.168.1.10 255.255.255.252

ip ospf 1 area 0

ipv6 address 2001:DB8:3::2/64

ipv6 ospf 1 area 0

negotiation auto

interface GigabitEthernet0/1/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/1/1

no ip address

shutdown

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router ospf 1

router-id 4.4.4.4

network 192.168.1.8 0.0.0.3 area 0

network 192.168.1.12 0.0.0.3 area 0

network 192.168.100.4 0.0.0.0 area 0

redistribute bgp 65002 subnets

ipv6 router ospf 1

router-id 4.4.4.4

redistribute bgp 65002

router bgp 65002

bgp router-id 4.4.4.4

bgp log-neighbor-changes

neighbor 192.168.1.9 remote-as 65002

neighbor 192.168.1.9 description Router3-IPv4-iBGP

neighbor 192.168.1.14 remote-as 65003

neighbor 192.168.1.14 description Router5-IPv4

neighbor 2001:DB8:3::1 remote-as 65002

neighbor 2001:DB8:3::1 description Router3-IPv6-iBGP

neighbor 2001:DB8:4::2 remote-as 65003

neighbor 2001:DB8:4::2 description Router5-IPv6

address-family ipv4

network 192.168.1.8 mask 255.255.255.252

network 192.168.1.12 mask 255.255.255.252

network 192.168.100.4 mask 255.255.255.255

redistribute ospf 1

neighbor 192.168.1.9 activate

neighbor 192.168.1.14 activate

exit-address-family

address-family ipv6

redistribute ospf 1

neighbor 2001:DB8:3::1 activate

neighbor 2001:DB8:4::2 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

login

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

line vty 5 15

login

Router 5:

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname Router5

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

no ip domain lookup

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 192.168.100.5 255.255.255.255

interface GigabitEthernet0/0/0

ip address 192.168.1.14 255.255.255.252

ipv6 address 2001:DB8:4::2/64

ipv6 eigrp 65003

negotiation auto

interface GigabitEthernet0/0/1

ip address 192.168.1.17 255.255.255.252

ipv6 address 2001:DB8:5::1/64

ipv6 eigrp 65003

negotiation auto

interface GigabitEthernet0/1/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/1/1

no ip address

shutdown

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router eigrp 65003

router-id 5.5.5.5

network 192.168.1.12 0.0.0.3

network 192.168.1.16 0.0.0.3

network 192.168.100.5 0.0.0.0

no auto-summary

redistribute bgp 65003 metric 10000 100 255 1 1500

ipv6 router eigrp 65003

eigrp router-id 5.5.5.5

redistribute bgp 65003 metric 10000 100 255 1 1500

router bgp 65003

bgp router-id 5.5.5.5

bgp log-neighbor-changes

neighbor 192.168.1.13 remote-as 65002

neighbor 192.168.1.13 description Router4-IPv4

neighbor 2001:DB8:4::1 remote-as 65002

neighbor 2001:DB8:4::1 description Router4-IPv6

address-family ipv4

network 192.168.1.12 mask 255.255.255.252

network 192.168.1.16 mask 255.255.255.252

network 192.168.100.5 mask 255.255.255.255

redistribute eigrp 65003

neighbor 192.168.1.13 activate

exit-address-family

address-family ipv6

redistribute eigrp 65003

neighbor 2001:DB8:4::1 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

login

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

line vty 5 15

login

Router 6:
version 16.9
service timestamps debug datetime msec
service timestamps log datetime msec
platform qfp utilization monitor load 80
platform punt-keepalive disable-kernel-core
hostname Router6
boot-start-marker
boot-end-marker
vrf definition Mgmt-intf
address-family ipv4
exit-address-family
address-family ipv6
exit-address-family
no aaa new-model
no ip domain lookup
login on-success log
subscriber templating
vtp domain cisco
vtp mode transparent
ipv6 unicast-routing
multilink bundle-name authenticated
diagnostic bootup level minimal
spanning-tree extend system-id
redundancy
mode none

interface Loopback0
ip address 192.168.100.6 255.255.255.255

interface GigabitEthernet0/0/0
no ip address
shutdown
negotiation auto

interface GigabitEthernet0/0/1
ip address 192.168.1.18 255.255.255.252
negotiation auto
ipv6 address 2001:DB8:5::2/64
ipv6 eigrp 65003

interface GigabitEthernet0/1/0
no ip address
shutdown
negotiation auto

interface GigabitEthernet0/1/1
no ip address
shutdown
negotiation auto

interface GigabitEthernet0
vrf forwarding Mgmt-intf
no ip address
shutdown
negotiation auto

router eigrp 65003
router-id 6.6.6.6
network 192.168.1.16 0.0.0.3
network 192.168.100.6 0.0.0.0
no auto-summary

ipv6 router eigrp 65003
eigrp router-id 6.6.6.6

ip forward-protocol nd
ip http server
ip http authentication local
ip http secure-server
ip tftp source-interface GigabitEthernet0

control-plane

line con 0
login
transport input none
stopbits 1

line aux 0
stopbits 1

line vty 0 4
login

line vty 5 15
login

end

Problems

This lab came with a ton of unexpected difficulties, mainly due to the many different layers involved in running multiple routing protocols concurrently, and making sure information was distributed properly. Because we used EIGRP, OSPF, and BGP at the same time with IPv4 and IPv6, we were especially susceptible to small errors that could lead to the network being unable to work.

The first issue was that there was missing routes between sub-networks. After I configured EIGRP on the outer networks and OSPF on the one between them, I thought I had finished the lab, but for some reason the loopback networks did not show up in the routing tables. Then, when I issued the command show ip route, I noticed that routes appeared in EIGRP but did not appear for the other protocols. Because of this discovery, I was able to troubleshoot and discover that redistribution had only been configured in one direction. I was able to fix this by adding redistribution commands to the other side, so router from EIGRP into BGP then into OSPF could flow smoothly. Show ip route and show ipv6 route only showed routes inside their sub-network which was how I knew the issue was redistribution.

Another big issue was the BGP neighbor adjacencies. The IPv4 BGP network came up, but the IPv6 routes were not being shared. When I issued the show bgp ipv6 unicast summary, it appeared empty. I then looked at the configuration and discovered that IPv6 neighbors must be specifically activated under the IPv6 address family and I had to do more than defining the neighbor globally. After I activated the IPv6 neighbors under the address family, the ipv6 routes started working correctly.

I then had issues with the EIGRP redistribution metrics. The BGP routes going into EIGRP were all missing or unreachable. This happened because EIGRP needs a complete metric to be defined during redistribution with another protocol. The first time I input the command, I forgot to include the metric values, which made EIGRP reject the routes. I solved this problem by inputting bandwidth, delay, reliability, load, and MTU values in the redistribution command. After doing this, the redistribution routes were then accepted and visible in the routing table.

Another challenge came when I wanted to verify OSPF adjacency. One of the routers in the OSPF part of the network was stuck in the wrong state. After I checked the configuration, I was able to realize that both routers were in area 0, but one of the interfaces was not correctly configured to run OSPF. Adding the missing OSPF command on the interface fixed the problem and allowed the adjacency to go to the FULL state

There were also times when routing loops or unexpected paths happened. Some routes preferred different protocols that I did not intend them to. I then examined the administrative distances and used show ip route to troubleshoot. Through this process, I was able to understand why the certain paths got chosen and I what was happening was not an error.

Lastly, troubleshooting in general took ridiculously long due to the huge amount of configs necessary on each router, as well as how foreign the some of the new commands we learned felt. An example of this was when we got IPv4 addressing to work, but when we moved to IPv6, it took a lot longer to achieve functionality. Solving these problems allowed me to refine my troubleshooting skills because I was able to practice skills like checking neighbor relationships, routing tables, redistribution and each router’s configurations step by step.

As a whole, lab needed lots of patience and steady troubleshooting. Most of the problems came from how the interfaces were configured, and sneaky commands that we were not aware of previously. Fixing these issues allowed me to better understand the interactions between protocols in real industry-style networks. I also learned the importance of each configuration and how one typo can be the difference between a functioning network and a broken one.

Conclusion

This lab helped me understand how different routing protocols become part of the same network. I saw firsthand how routes move to other protocols and how important redistribution specificity is. The lab also improved my troubleshooting a ton, specifically when dealing with missing routes and connectivity issues. As a whole, this lab was valuable because it helped me gain real world experience by allowing me to design a network designed similar to those used by real companies.